Electrical Machine AI Lab

Experiment - 2

Introduction

Welcome to the experiment “To Study the Load Characteristics of a DC Shunt Generator.” In this experiment, you will learn how to determine and analyze the load characteristics of a DC shunt generator, and how to plot both the internal and external characteristics under varying load conditions.

To guide you throughout the learning process, this experiment is organized into several key sections. We begin with the Aim, which clearly states what you will learn and why the experiment is important. This is followed by the Theory section, where you will find all the essential concepts and foundational knowledge required to understand the working and behaviour of a DC shunt generator.

Before you proceed to the simulation, the Pretest section helps you evaluate your basic understanding of the topic. Once you have attempted the MCQ questions in the Pretest, you can move on to the Procedure, which outlines each step you need to follow to perform the experiment correctly. The most important part is the Simulation, where you can use the virtual setup and perform the experiment yourself.

After completing the simulation, the Posttest section allows you to check what you have learned after performing the experiment. Finally, the References and Contributors sections provide the supporting resources and acknowledge the team behind the development of this experiment.

Together, these sections ensure a smooth, structured, and engaging learning experience from start to finish.

Experiment name: 
To Study the Load Characteristics of a DC Shunt Generator.

Aim

To study the load characteristics of DC Shunt Generator. Draw the internal characteristics and external characteristics under different loading conditions.

Theory

### Introduction

A DC shunt generator is a type of self-excited DC generator in which the field winding is connected in parallel (shunt) with the armature winding. As a result, the field current remains almost constant (since it depends only on the terminal voltage). The yoke of a DC generator is made of cast iron because it provides high magnetic permeability, good mechanical strength, and serves as a low-reluctance path for the magnetic flux.

The performance of a DC shunt generator is described using load characteristics, which illustrate how the terminal voltage and load current change as the load varies.

In a shunt generator, the field winding is connected in parallel with the armature winding so that the terminal voltage of the generator is applied across it. The shunt field winding has many turns of fine wire having high resistance. Therefore, only a part of the armature current flows through the shunt field winding, and the rest flows through the load. Fig. 1 shows the connections of a shunt wound generator. The armature current Ia splits up into two parts - a small fraction Ish flowing through the shunt field winding, while the major part IL goes to the external load.

Fig. 1 Equivalent circuit of DC shunt generator for load characteristics

### Losses in a DC Shunt Generator

In a DC shunt generator, various losses occur during operation, such as armature copper losses, shunt field copper losses, iron losses, and mechanical losses. Among these, armature copper losses constitute a significant portion of the total losses. The copper losses in the armature of a DC shunt generator are approximately 30% of the total full-load losses. These losses occur due to the resistance of the armature winding and increase with load current, thereby affecting the efficiency and voltage regulation of the generator.

Types of Characteristics

There are two main characteristics of a DC shunt generator:

- Internal characteristic

The internal characteristic curve represents the relation between the generated voltage (Eg) and the load current IL. When the generator is loaded, the generated voltage decreases due to armature reaction. Therefore, the generated voltage will be lower than the emf generated at no load. Here, in Fig. 2, the AD curve shows the no-load voltage curve, and AB represents the internal characteristic curve.

Fig. 2 Characteristics of DC shunt generator

- External or Load characteristic

The external characteristic of a DC shunt generator represents the relationship between the terminal voltage (V) and the load current (IL) while the generator is supplying power. It shows how the terminal voltage varies with changes in the load.

This characteristic is typically represented by a V–IL graph. In Fig. 2, the AC curve represents the external characteristic of the DC shunt generator, showing the variation of terminal voltage with load current. Due to the ohmic drop in the armature resistance, the terminal voltage is lower than the generated voltage. Therefore, the external characteristic curve lies below the internal characteristic curve.

The terminal voltage is given by:

V = (Eg - Ia Ra) = Eg - (IL + Ish)Ra

Where, V is the terminal voltage of the generator

Eg is the generated emf in the armature

Ia is armature current, and Ra is armature resistance

Ish is the current flowing through the shunt field winding

IL is the load current delivered to the external load connected to the generator

### Behaviour of External Characteristics

When the load resistance of a DC shunt generator decreases, the load current (IL) increases, as shown in the above figure. However, this increase in current is limited only up to point C, known as the breakdown point. Beyond this point, a further decrease in load resistance causes a decrease in load current, and the external characteristic curve reverses (shown by the dotted line). Eventually, the terminal voltage may drop to zero, except for a small value due to residual magnetism. The point at which the generator delivers maximum current is called the breakdown point (point C in the graph).

Advantages:

1. Provides nearly constant terminal voltage under light to medium load.
2. Stable operation within a certain load range.

3. Suitable for battery charging, lighting, and excitation purposes where steady voltage is required.

4. Simple voltage control by adjusting the field current.

Disadvantages:

1. Poor voltage regulation at high load current.

2. Stability decreases with decreasing load resistance.

3. Depends on residual magnetism for initial voltage build-up.

4. Breakdown point limitation: beyond a certain load current, the characteristic curve reverses.

Procedure

STEP 1: Make Connections as per the instructions given below:

| From | R | R | B | B | A2 | L | F | A | L2 | A4 | Z4 | I | J | H | H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| To | C | E | G | A2 | Z2 | D | Z1 | A1 | A4 | Z4 | K | J | L1 | A3 | Z3 |

STEP 2: Click on the "Check" Button to check the connections.

STEP 3: If connections are correct, the Supply will be turned ON.

STEP 4: Select the No. of Bulbs from the Lamp load.

STEP 5: Click on the "Add" Button to add the values to the Observation Table.

STEP 6: Add different values to the Table by increasing the lamp load.

STEP 7: Click on the "Graph" Button to Create a Graph.

STEP 8: Click on the "Print" Button to print the webpage.

STEP 9: Click on the "Reset" Button to reset the webpage.

Pretest

1. A DC shunt generator has a terminal voltage versus load current characteristic which is

Answer:  b: Slightly drooping

2. The terminal voltage of a DC shunt generator drops with load because of

Answer:   d: All of these

3. In a DC shunt generator, the shunt field current mainly depends on:

Answer: b) Terminal voltage

4. When a DC shunt generator is loaded, which loss increases significantly with load current?

Answer: c) Armature copper loss

5. The voltage regulation of a DC shunt generator is defined as:

Answer: c) Percentage change in terminal voltage from no-load to full-load

6. The main function of a DC generator is to:

Answer: b) Convert mechanical energy into electrical energy

7. In a DC shunt generator, the field winding is connected:

Answer: b) In parallel with the armature

8. The emf generated in a DC generator is given by:

Answer: b) E = kΦN

9. In a DC generator, the armature reaction causes:

Answer: b) Decrease in flux

10. The load characteristic of a DC shunt generator is the relation between:

Answer: b) Terminal voltage and load current

11. As the load current increases in a DC shunt generator, the terminal voltage:

Answer: b) Decreases

12. The drop in terminal voltage under load is mainly due to

Answer: a) Armature reaction and armature resistance

13. In a DC shunt generator, residual magnetism is required for:

Answer: b) Self-excitation

14. The open circuit characteristic (OCC) of a DC generator represents:

Answer: a) Voltage vs field current at constant speed

15. The internal characteristic of a DC generator shows the relation between:

Answer: c) Generated emf and armature current

16. The external characteristic of a DC generator shows the relation between:

Answer: b) Terminal voltage and load current

17. Which of the following losses occur in a DC generator?

Answer: d) All of the above

18. The load test on a DC shunt generator is performed to determine:

Answer: c) Load characteristics

19. In a DC shunt generator, field current is:

Answer: a) Constant for a given supply voltage

20. The terminal voltage of a DC shunt generator on full load is:

Answer: c) Less than emf generated

Posttest

1. Armature magnetic field in a dc generator produces which of the following effect?

Answer: d: Both (a) and (b)

2.Copper losses in the armature of the DC shunt generator amount to which of the following percentages of full load losses?   —-----

Answer: c) 20 to 30%

3. In a DC generator, compared to the air gap under the field poles, the interpole air gap is made

Answer:a: Larger

4. If the field current of a DC shunt generator is slightly reduced during operation, the    terminal voltage will:

Answer: b) Decrease gradually

The yoke of a dc generator is made of cast iron because

Answer:d: All of these

6. What happens to the terminal voltage of a DC shunt generator as load increases?

Answer: b) It decreases

7. The main reason for the drop in terminal voltage under load is:

Answer: a) Armature resistance drop and armature reaction

8. What does the external characteristic curve of a DC shunt generator represent?

Answer: b) V vs IL

9. If the terminal voltage of the generator falls sharply with increasing load, it indicates:

Answer: b) Poor voltage regulation

10. How can the voltage regulation of a DC shunt generator be improved?

Answer: b) By increasing field excitation

11. In a well-designed DC shunt generator, the voltage drop from no-load to full-load is typically:

Answer: b) Around 5–10%

12. During the experiment, the load on the DC shunt generator is varied by:

Answer: c) Varying the load resistance connected across the armature

13. The reason for using a shunt field regulator in the experiment is to:

Answer: b) Vary the excitation current

14. When load increases, armature current increases. This causes:

Answer: a) More voltage drop across armature resistance

15. The generated emf in the generator is directly proportional to:

Answer: c) Speed and field flux

16. In the experiment, the field current is kept constant to obtain:

Answer: b) Load characteristic

17. The curve plotted between terminal voltage and load current is generally:

Answer: b) Slightly drooping curve

18. In the experiment, if the speed of the generator fluctuates, it will:

Answer: b) Cause incorrect results in load characteristics

19. The load characteristics help determine:

Answer: a) Efficiency and voltage regulation

20. Which safety precaution is essential while performing this experiment?

Answer: b) Never exceed the rated load current

- References

- N. Mohan, Electric Machines and Drives: A First Course, Wiley, 2012.

- A. Veltman, D.W.J. Pulle, and R.W. DeDoncker, Advanced Electrical Drives: Analysis, Modeling, Control, Springer, 2011.

- J.L. Kirtley, Electric Power Principles: Sources, Conversion, Distribution, and Use, Wiley, 2010.

- A. Veltman, D.W.J. Pulle, and R.W. DeDoncker, Fundamentals of Electrical Drives, Springer, 2007.

- I. Boldea and S.A Nasar, Electric Drives, CRC Press, 2nd ed. 2006.

- J. Chiasson, Modeling and High Performance Control of Electric Machines, Wiley-IEEE, 2005.

- P.C. Krause, O. Wasynczuk, and S.D. Sudhoff, Analysis of Electric Machinery and Drive Systems,IEEE Press, 2nd ed. 2002.

- B. Amin, Induction Motors: Analysis and Torque Control, Springer, 2002.

- N. Mohan, Advanced Electric Drives: Analysis, Control and Modeling using Simulink , published by MNPERE (www.MNPERE.com), 2001.

- W. Leonhard, Control of Electrical Drives, Springer, 3rd ed. 2001.

- Simulation

Apparatus - The simulation includes the following apparatus:

MCB: An MCB (Miniature Circuit Breaker) is an automatically operated electrical switching device used to protect an electrical circuit from overcurrent, short circuit, or excessive load. It acts as a safety device and disconnects the circuit when abnormal conditions occur.

MCB acts as the main supply switch for the entire experiment setup. It protects the setup from damage during faults or wrong connections. If any excessive current flows, it trips and stops power, ensuring the safety of instruments and users.

3-Point Starter: A 3-point starter is a device used to safely start DC shunt and compound motors. It limits the starting current and provides overload protection to the motor. It ensures smooth acceleration by gradually reducing resistance as the motor gains speed, providing a safe start.

Connections: L - Line / Supply , F - Field winding of motor, A - Armature of motor

Detailed Explanation: A 3-point starter works to safely start a DC shunt or compound motor by gradually applying voltage and limiting the initial high current. Its main components include the starter handle, which is moved manually from the OFF position to the RUN position, a series of resistances that limit the starting current and are gradually removed as the handle moves forward the No-Volt Coil (NVC), which holds the handle in the RUN position using magnetic force and protects the motor by releasing the handle if the supply voltage fails or field current drops and the Overload Relay (OLR), which disconnects the motor if the current exceeds safe limits.

While working, the handle is slowly moved forward, progressively cutting out the resistance and allowing the motor to gain speed smoothly. Once at full speed, the handle reaches the RUN position where the NVC keeps it in place. If overload or voltage failure occurs, the NVC or OLR releases the handle, automatically disconnecting the motor from the supply and preventing damage, ensuring safe and protected operation.

DC Motor: In this experiment, the DC motor acts as a prime mover, converting electrical energy into mechanical energy. Its shaft is mechanically coupled to the DC shunt generator, and it receives the supply through the MCB and starter.

DC Shunt Generator: The DC shunt generator converts the mechanical energy received from the motor into electrical energy, producing DC output voltage and current, which is then supplied to the lamp load, while its terminals are connected to an ammeter in series and a voltmeter in parallel for measurement.

DC Ammeter 1: This ammeter is connected in series with the DC Motor. Its purpose is to measure the current drawn by the DC motor while it is operating.

DC Voltmeter 1: It is connected in parallel across the MCB. The purpose of this voltmeter is to measure the voltage of the main supply.

DC Ammeter 2: It is connected in series with the lamp load to measure the load current (IL) delivered by the DC shunt generator.

DC Voltmeter 2: It is connected in parallel across the generator terminals to measure the terminal voltage (V) of the DC shunt generator.

Lamp Load: In this experiment, a lamp load is used as a variable resistive load to study the load characteristics of a DC shunt generator. It helps in observing how the terminal voltage varies with the load current.

The bulbs can be switched ON or OFF to increase or decrease the load resistance and control the load current. More bulbs ON produce higher current due to lower resistance, while fewer bulbs result in lower current due to higher resistance. These variations in load are used to record terminal voltage and load current readings, which are then plotted to obtain the V–I characteristic curve.

Ratings:

3-Point Starter: Voltage - 220V DC, 7.5 HP

Range of DC Voltmeter 1:   0 - 240 V

Range of DC Voltmeter 2:   0 - 240 V

Range of DC Ammeter 1:   0 - 20 A

Range of DC Ammeter 2:   0 - 20 A

DC Motor Ratings: 5HP, Voltage - 220 V DC, Max. Current - 19 A, Speed - 1500 RPM

Winding Type - Shunt

DC Shunt Generator: 3 kW, Voltage - 220 V DC, Max. Current - 13.6 A, Speed - 1500 RPM

Lamp Load: 2 kW (Each bulb has a rating of 200 W).

- About buttons:

Instructions: The Instructions provide step-by-step guidance on how to perform the experiment.

Check: The Check button verifies all the connections to ensure that they are correct.

Reset: The Reset button refresh the experiment to its initial state for a fresh start.

Add: The Add button adds the readings to the observation table.

Graph: The Graph button helps visualize the readings through a graph.

Print: The Print button allows you to print the simulation.