### PRETEST

### A DC shunt generator has a terminal voltage versus load current characteristic which is

### Answer:  b: Slightly drooping

### The terminal voltage of a DC shunt generator drops with load because of

### Answer:   d: All of these

### 3 . In a DC shunt generator, the shunt field current mainly depends on:

### Answer: b) Terminal voltage

### 4. When a DC shunt generator is loaded, which loss increases significantly with load current?

### Answer: c) Armature copper loss

### 5. The voltage regulation of a DC shunt generator is defined as:

### Answer: c) Percentage change in terminal voltage from no-load to full-load

### 6. The main function of a DC generator is to:

a) Convert electrical energy into mechanical energy
 b) Convert mechanical energy into electrical energy
 c) Step up voltage
 d) Step down current
 Answer: b) Convert mechanical energy into electrical energy

### 7. In a DC shunt generator, the field winding is connected:

a) In series with the armature
 b) In parallel with the armature
 c) Across the load
 d) In series with the load
 Answer: b) In parallel with the armature

### 8. The emf generated in a DC generator is given by:

a)  E = V + Ia Ra
 b) E = kΦN
 c) E = V/I 
 d) E = P/I 
 Answer: b) E = kΦN

### 9. In a DC generator, the armature reaction causes:

a) Increase in flux
 b) Decrease in flux
 c) No effect on flux
 d) Constant emf
 Answer: b) Decrease in flux

### 10. The load characteristic of a DC shunt generator is the relation between:

a) Armature current and field current
 b) Terminal voltage and load current
 c) Load current and speed
 d) Speed and torque
 Answer: b) Terminal voltage and load current

### 11. As the load current increases in a DC shunt generator, the terminal voltage:

a) Increases
 b) Decreases
 c) Remains constant
 d) Becomes zero
 Answer: b) Decreases

### 12. The drop in terminal voltage under load is mainly due to:

a) Armature reaction and armature resistance
 b) Commutator resistance only
 c) Eddy current loss only
 d) Field resistance
 Answer: a) Armature reaction and armature resistance

### 13. In a DC shunt generator, residual magnetism is required for:

a) Smooth commutation
 b) Self-excitation
 c) Speed control
 d) Field current regulation
 Answer: b) Self-excitation

### 14. The open circuit characteristic (OCC) of a DC generator represents:

a) Voltage vs field current at constant speed
 b) Voltage vs load current at variable speed
 c) Current vs resistance
 d) Speed vs torque
 Answer: a) Voltage vs field current at constant speed

### 15. The internal characteristic of a DC generator shows the relation between:

a) Generated emf and field current
 b) Terminal voltage and field current
 c) Generated emf and armature current
 d) Terminal voltage and armature current
 Answer: c) Generated emf and armature current

### 16. The external characteristic of a DC generator shows the relation between:

a) Generated emf and field current
 b) Terminal voltage and load current
 c) Generated emf and armature current
 d) Torque and speed
 Answer: b) Terminal voltage and load current

### 17. Which of the following losses occur in a DC generator?

a) Copper losses
 b) Iron losses
 c) Mechanical losses
 d) All of the above
 Answer: d) All of the above

### 18. The load test on a DC shunt generator is performed to determine:

a) Armature efficiency
 b) Speed regulation
 c) Load characteristics
 d) Field efficiency
 Answer: c) Load characteristics

### 19. In a DC shunt generator, field current is:

a) Constant for a given supply voltage
 b) Variable with speed
 c) Dependent on load current
 d) Zero at all times
 Answer: a) Constant for a given supply voltage

### 20. The terminal voltage of a DC shunt generator on full load is:

a) Equal to emf generated
 b) Greater than emf generated
 c) Less than emf generated
 d) Independent of emf generated
 Answer: c) Less than emf generated

Posttest

### 1. Armature magnetic field in a DC generator produces which of the following effect?

a: It demagnetizes or reduces the main flux
b: It cross-magnetizes the main flux
c: It magnetizes or reinforces the main flux
d: Both (a) and (b)

Correct answer: d

2. Copper losses in the armature of the DC shunt generator amount to which of the following percentages of full load losses?   —-----

a) 5 to 10%

b) 10 to 20%

c) 20 to 30%

d) 30 to 40%

Correct answer:  c

### 3. In a DC generator, compared to the air gap under field poles, the interpole air gap is made:

a: Larger
b: Smaller
c: The same
d: Much smaller

Correct answer: a

4. If the field current of a DC shunt generator is slightly reduced during operation, the terminal voltage will:

a) Increase sharply

b) Decrease gradually

c) Remain unchanged

d) Become zero
Correct answer: b

### 5. The yoke of a DC generator is made of cast iron because:

a: It is cheaper
b: It completes the magnetic path
c: It gives mechanical protection to the machine
d: All of these

Correct answer: d

6. What happens to the terminal voltage of a DC shunt generator as load increases?

a) It increases
 b) It decreases
 c) It remains constant
 d) It first increases, then decreases
 Answer: b) It decreases

### 7. The main reason for the drop in terminal voltage under load is:

a) Armature resistance drop and armature reaction
 b) Decrease in field resistance
 c) Increase in field current
 d) Decrease in speed
 Answer: a) Armature resistance drop and armature reaction

### 8. What does the external characteristic curve of a DC shunt generator represent?

a) E vs Ia
 b) V vs IL
 c) E vs If
 d) V vs N
 Answer: b) V vs IL

### 9. If the terminal voltage of the generator falls sharply with increasing load, it indicates:

a) High armature resistance
 b) Poor voltage regulation
 c) Weak residual magnetism
 d) Low field current
 Answer: b) Poor voltage regulation

### 10. How can the voltage regulation of a DC shunt generator be improved?

a) By increasing armature resistance
 b) By increasing field excitation
 c) By reducing the field current
 d) By increasing speed
 Answer: b) By increasing field excitation

### 11. In a well-designed DC shunt generator, the voltage drop from no-load to full-load is typically:

a) Negligible
 b) Around 5–10%
 c) 30–40%
 d) 50–60%
 Answer: b) Around 5–10%

### 12. During the experiment, the load on the DC shunt generator is varied by:

a) Changing the armature speed
 b) Adjusting the rheostat in the field circuit
 c) Varying the load resistance connected across the armature
 d) Changing supply voltage
 Answer: c) Varying the load resistance connected across the armature

### 13. The reason for using a shunt field regulator in the experiment is to:

a) Protect the generator
 b) Vary the excitation current
 c) Maintain constant load
 d) Reduce armature reaction
 Answer: b) Vary the excitation current

### 14. When load increases, armature current increases. This causes:

a) More voltage drop across armature resistance
 b) More field flux
 c) Less terminal current
 d) Constant emf
 Answer: a) More voltage drop across armature resistance

### 15. The generated emf in the generator is directly proportional to

a) Load current
 b) Armature resistance
 c) Speed and field flux
 d) Terminal voltage
 Answer: c) Speed and field flux

### 16. In the experiment, the field current is kept constant to obtain:

a) Open circuit characteristics
 b) Load characteristics
 c) Internal characteristics
 d) Magnetization curve
 Answer: b) Load characteristics

### 17. The curve plotted between terminal voltage and load current is generally:

a) Straight line
 b) Slightly drooping curve
 c) Parabolic
 d) Exponential
 Answer: b) Slightly drooping curve

### 18. In the experiment, if the speed of the generator fluctuates, it will:

a) Have no effect on voltage
 b) Cause incorrect results in load characteristics
 c) Reduce field resistance
 d) Increase copper loss
 Answer: b) Cause incorrect results in load characteristics

### 19. The load characteristics help determine:

a) Efficiency and voltage regulation
 b) Field winding design
 c) Commutator dimensions
 d) Mechanical losses
 Answer: a) Efficiency and voltage regulation

### 20. Which safety precaution is essential while performing this experiment?

a) Start the generator without excitation
 b) Never exceed the rated load current
 c) Disconnect the load suddenly
 d) Operate at maximum speed
 Answer: b) Never exceed the rated load current